# Benchmark: application of a pore pressure traction

# Boundary conditions

A pressure traction of -10kPa is applied at the top surface. Load is applied at the NODES.

| Figure 1 pressure traction | Figure 2 Solid fixity | Figure 3 Liquid fixity |
| --- | --- | --- |

# Material parameters

$$MATERIAL_NAME

Soil

$$MATERIAL_TYPE

2-phase

$$POROSITY_SOLID

0.40

$$DENSITY_SOLID

2650

$$K0_VALUE_SOLID

0.5

$$INTRINSIC_PERMEABILITY_LIQUID

1e-10

$$DENSITY_LIQUID

1000

$$BULK_MODULUS_LIQUID

20000.0

$$DYNAMIC_VISCOSITY_LIQUID

1e-6

$$MATERIAL_MODEL_SOLID

linear_elasticity

$$YOUNG_MODULUS

1000

$$POISSON_RATIO

0.3

# Discretization

4MP per elements are used. The mesh counts 30 tetrahedral elements.

# Load phases

- Load phase 1: gravity loading, apply quasi-static convergence. Multiplier liquid = 0, damping factor 0.75

- Load phase 2-5: pressure loading. Multiplier liquid = 1, damping factor=0, total time=2s:

$$NUMBER_OF_LOADSTEPS

11

$$TIME_PER_LOADSTEP

2.000000000000000E-001

$$COURANT_NUMBER

0.900000000000000

$$COMPUTATION_METHOD

MPM-MIXED

$$NUMBER_OF_PHASES

2

$$NUMBER_OF_POINTS

1

$$GRAVITY_VECTOR

0.000000000000000E+000  -1.00000000000000       0.000000000000000E+000

$$MULTIPLIER_GRAVITY

1.00000000000000        1.00000000000000

$$MULTIPLIER_LIQUID

1.000000000000000E+000  1.000000000000000E+000

$$LOAD_TYPE

STEP

# Results

| Figure 4 pore pressure at load step 1 (hydrostatic distribution) | Figure 5 pore pressure at load step 10 |
| --- | --- |

Figure 6 Pressure at material points during load step 1 (left) and load steps 2-11 (right).

# Conclusions

The pore pressure increases with time as expected.

Modifications conducted on 11 August, 2021:

In order to reproduce this benchmark using the opensource (version 2021), a new GiD file was created. This subsequently changed the material point (MP) numbers. Table 1 provides the new MP ID corresponding to the Y coordinate in the old MP. Figure 6 and 7 show the reproduced results using the opensource for the dynamic and quasi-static portions of the simulation.

Table 1: Reference MP ID numbers for the newly benchmark for the same problem

| Y-Coordinate (m) | Old MP ID | New MP ID |
| --- | --- | --- |
| 0.33 | 2 | 51 |
| 0.24 | 16 | 11 |
| 0.14 | 28 | 8 |
| 0.0862 | 46 | 2 |

Figure 6: Pore pressure result obtained for the dynamic part

Figure 7: Pore pressure result obtained for the quasi-static part