Oedometer_QS_ULFEM_1Phase_NormalStress – written by Gaia Di Carluccio, 21-10-2021

General description

In this benchmark a 2D column of soil of 1m height is loaded at the top with a vertical load of 20kPa. UL-FEM computational method and quasi-static calculation is used. The soil consists of 1-phase (solid) with a linear-elastic model.

Mesh

The initial column is 0.1 m x 1 m.

Type of element: triangular-3 nodes

Number of element: 22

Size of element: 0.1

Number of nodes: 24

Material properties

$$MATERIAL_TYPE

dry_material

$$POROSITY_SOLID

0.3

$$DENSITY_SOLID

2600

$$K0_VALUE_SOLID

0.0

$$MATERIAL_MODEL_SOLID

linear_elasticity

$$YOUNG_MODULUS

100

$$POISSON_RATIO

0.2

Boundary conditions

The top of the column is fixed in y direction, the bottom is fixed in x and y directions and the sides of the column are fixed in lateral direction.

A vertical load of -20 kPa is applied at the top line of the soil column.

Gravity

Gravity is not applied.

Calculation data

### Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

1

$$TIME_PER_LOADSTEP

0.1

$$TOTAL_TIME

0.0

$$COURANT_NUMBER

0.8

$$COMPUTATION_METHOD

UL-FEM

$$GRAVITY_ACCELERATION

9.81

$$GRAVITY_VECTOR

0.0 -1.0 0.0

$$GRAVITY_LOAD

off  0.0  1.0

$$SOLID_TRACTION

step  0.0  1.0

$$LIQUID_PRESSURE

off  0.0  1.0

$$PRESCRIBED_VELOCITY

off  0.0  1.0

$$QUASISTATIC_CONVERGENCE

1

$$TOLERATED_ERROR_SOLID

0.001 0.001

$$TOLERATED_ERROR_LIQUID

0.001 0.001

$$MAXIMUM_TIME_STEPS

10000

$$HOMOGENEOUS_LOCAL_DAMPING

1 0.75

$$OUTPUT_NUMBER_OF_NODES

10

$$OUTPUT_NODES

4

5

8

9

12

13

16

17

20

21

$$END