Benchmark Test: Gravitation_DYN_MPMMIXED_2Phase_Gravity_Dynamic_2D:

written by Luis A. Aviles, 02/07/2021

General description

A 2D dry soil column, with elastic behaviour, is loaded using gravity load apply instantly from first time step. Bottom is fixed and the sample can only deform in vertical direction.

Mesh

Size of mesh: 0,1 m x 1,1 m x 0,1 m

Type of element: triangular_3-noded

Number of elements: 22

Size of elements: 0.1

Number of nodes: 24

Number of MPs per element: 3 SMPs

Material properties

$$MATERIAL_NAME

Elastic

$$MATERIAL_TYPE

saturated_material_coupled

$$POROSITY_SOLID

0.25

$$DENSITY_SOLID

2650

$$DENSITY_LIQUID

1000

$$INTRINSIC_PERMEABILITY_LIQUID

1e-9

$$BULK_MODULUS_LIQUID

10000

$$DYNAMIC_VISCOSITY_LIQUID

1e-6

$$K0_VALUE_SOLID

0.0

$$MATERIAL_MODEL_SOLID

linear_elasticity

$$YOUNG_MODULUS

10000

$$POISSON_RATIO

0.3

Boundary conditions

The displacement for solid and liquid at the boundaries will be ﬁxed in normal direction and free in the tangential directions

Initial conditions

No initial conditions

Gravity

Gravity load is applied in the first-time step.

Numerical Solution:

### Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

10

$$TIME_PER_LOADSTEP

0.2

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_LOAD

step  1.0  1.0

$$SOLID_TRACTION

off  0.0  1.0

$$MASS_SCALING

0 1.0

$$HOMOGENEOUS_LOCAL_DAMPING

0 0.75

$$FIX_SOLID_SKELETON

0

$$STRAIN_SMOOTHING

0

$$APPLY_OBJECTIVE_STRESS

0

$$DEGREE_OF_FILLING

0.9

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

6

$$OUTPUT_MATERIAL_POINTS

1

10

22

34

46

58

$$END

The output is given for 6 material point. From the .PAR file it is possible to plot time steps (or time) versus load multiplier.

58

46

34

22

10

1

Analytical Solution

No analytical solution