Benchmark Test: 2D prescribed velocity at nodes on line

written by Francesca Ceccato, 05/11/2018; revised by Kaleigh Yost, 06/08/2021

General description

The benchmark considers the penetration of a footing into the soil. Stresses are initialized with K0 procedure and the foundation penetrates at a prescribed velocity v0. The moving mesh option is applied, the fixed mesh is the portion including the footing. The load multipliers for velocity are applied.

Mesh

Figure 1 geometry and discretization

Size of elements: 0.5m

$$DIMENSION

2D-plane_strain

$$ELEMENTTYPE

triangular_3-noded

Num. of elements= 910

Num. of nodes=1706

Number of MPs per element: 3 SMPs

Material properties

| foundation | soil |
| --- | --- |
| $$MATERIAL_TYPE dry_material $$POROSITY_SOLID 0.0 $$DENSITY_SOLID 2000 $$K0_VALUE_SOLID 0.0 $$MATERIAL_MODEL_SOLID linear_elasticity $$YOUNG_MODULUS 50000 $$POISSON_RATIO 0.33 $$MATERIAL_INDEX 2 | $$MATERIAL_TYPE dry_material $$POROSITY_SOLID 0.4 $$DENSITY_SOLID 2000 $$K0_VALUE_SOLID 0.5 $$MATERIAL_MODEL_SOLID mohr_coulomb $$YOUNG_MODULUS 20000 $$POISSON_RATIO 0.33 $$FRICTION_ANGLE 30 $$COHESION 0.5 $$DILATANCY_ANGLE 0.0 $$TENSILE_STRENGTH 0.0 |

Fixities

The left and right boundaries are fixed in the x-direction. The top and bottom boundaries are fixed in the y-direction.

Boundary conditions

The velocity of -0.5m/s is prescribed only in y-direction (x-direction is free) at the nodes of the top line of the foundation.

Figure 2 Prescribed velocity specifications

Moving mesh is applied:

Figure 3 Moving mesh specifications

Loading conditions

No traction is applied, gravity is applied at the first time step.

Calculation parameters:

### Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

10

$$TIME_PER_LOADSTEP

0.10

$$TOTAL_TIME

0.0

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_ACCELERATION

9.81

$$GRAVITY_VECTOR

0.0 -1.0 0.0

$$GRAVITY_LOAD

step  1.0  1.0

$$SOLID_TRACTION

off  1.0  100.0

$$LIQUID_PRESSURE

off  0.0  1.0

$$PRESCRIBED_VELOCITY

step  0.0  1.0

$$QUASISTATIC_CONVERGENCE

0

$$TOLERATED_ERROR_SOLID

0.01 0.05

$$TOLERATED_ERROR_LIQUID

0.01 0.01

$$MAXIMUM_TIME_STEPS

10000

$$MASS_SCALING

0 1.0

$$HOMOGENEOUS_LOCAL_DAMPING

0 0.7

$$BULK_VISCOSITY_DAMPING

0 0.06 1.2

$$FIX_SOLID_SKELETON

0

$$STRAIN_SMOOTHING

1

$$CONTACT_FORMULATION

0

$$INITIAL_WATER_PRESSURE

0.0

$$APPLY_POROSITY_UPDATE

0

$$INITIAL_VELOCITY

0

$$RESET_DISPLACEMENTS

0

$$REMOVE_FIXITIES

0 0 0

$$K0_PROCEDURE

1

$$SURFACE_ELEVATION

10.0

$$INITIAL_VERTICAL_LOAD_K0

0.0

$$EXCAVATION

0

$$SUBMERGED_CALCULATION

0 1

$$APPLY_OBJECTIVE_STRESS

1

$$DEGREE_OF_FILLING

0.9

$$NUMBER_OF_ACTIVE_ELEMENTS

0

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

6

$$OUTPUT_MATERIAL_POINTS

1

50

100

200

300

400

$$END

Solution:

Figure 2 shows the vertical displacement of a MP of the foundation (#1). The velocity is increases stepwise. The displacement follows the theoretical expectations. The velocity at MP of the foundation is -0.5m/s.  The moving mesh moves as expected.

Figure 4 Vertical displacement of a MP of the foundation

Figure 5 Vertical velocity at t=1s

Figure 6 Vertical displacement at t=1s