Benchmark Test: 2D shallow foundation step load on MPs

written by Kaleigh Yost, 14/07/2021

General description

The benchmark considers the penetration of a footing into the soil. Stresses are initialized with K0 procedure and the foundation penetrates with an applied load on the MPs. The moving mesh option is applied, the fix mesh is the portion including the footing. Load multipliers are applied.

Mesh

Figure 1 geometry and discretization

Size of elements: 0.5m

$$DIMENSION

2D-plane_strain

$$ELEMENTTYPE

triangular_3-noded

Num. of elements= 910

Num. of nodes=1706

Number of MPs per element: 3 SMPs

Material properties

| foundation | soil |
| --- | --- |
| $$MATERIAL_TYPE dry_material $$POROSITY_SOLID 0.0 $$DENSITY_SOLID 2000 $$K0_VALUE_SOLID 0.0 $$MATERIAL_MODEL_SOLID linear_elasticity $$YOUNG_MODULUS 50000 $$POISSON_RATIO 0.33 $$MATERIAL_INDEX 2 | $$MATERIAL_TYPE dry_material $$POROSITY_SOLID 0.4 $$DENSITY_SOLID 2650 $$K0_VALUE_SOLID 0.5 $$MATERIAL_MODEL_SOLID mohr_coulomb $$YOUNG_MODULUS 20000 $$POISSON_RATIO 0.33 $$FRICTION_ANGLE 30 $$COHESION 0.5 $$DILATANCY_ANGLE 0.0 $$TENSILE_STRENGTH 0.0 |

Fixities

The left and right boundaries are fixed in the x-direction. The bottom boundary is fixed in the x- and y-directions. The top boundary is fixed in the y-direction.

Figure 2 Fixities

Moving mesh is applied:

Figure 3 Moving mesh specifications

Loading conditions

Gravity is applied at the first time step. 100 kPa of vertical traction is applied to the MPs on the top of the foundation. Traction is increased via multipliers step-wise over each time step.

Figure 4. Loading conditions

Calculation parameters:

### Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

6

$$TIME_PER_LOADSTEP

1.0

$$TOTAL_TIME

0.0

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_ACCELERATION

9.81

$$GRAVITY_VECTOR

0.0 -1.0 0.0

$$GRAVITY_LOAD

step  1.0  1.0

$$SOLID_TRACTION

step  1.0  10.0

$$LIQUID_PRESSURE

off  0.0  1.0

$$PRESCRIBED_VELOCITY

off  1.0  1.0

$$QUASISTATIC_CONVERGENCE

1

$$TOLERATED_ERROR_SOLID

0.01 0.05

$$TOLERATED_ERROR_LIQUID

0.01 0.01

$$MAXIMUM_TIME_STEPS

10000

$$MASS_SCALING

0 1.0

$$HOMOGENEOUS_LOCAL_DAMPING

1 0.7

$$BULK_VISCOSITY_DAMPING

0 0.06 1.2

$$FIX_SOLID_SKELETON

0

$$STRAIN_SMOOTHING

1

$$CONTACT_FORMULATION

0

$$INITIAL_WATER_PRESSURE

0.0

$$APPLY_POROSITY_UPDATE

0

$$INITIAL_VELOCITY

0

$$RESET_DISPLACEMENTS

0

$$REMOVE_FIXITIES

0 0 0

$$K0_PROCEDURE

1

$$SURFACE_ELEVATION

10.0

$$INITIAL_VERTICAL_LOAD_K0

0.0

$$EXCAVATION

0

$$SUBMERGED_CALCULATION

0 1

$$APPLY_OBJECTIVE_STRESS

0

$$DEGREE_OF_FILLING

0.9

$$NUMBER_OF_ACTIVE_ELEMENTS

0

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

6

$$OUTPUT_MATERIAL_POINTS

1

2000

2500

3000

3957

4500

$$END

Solution

The vertical displacement of a foundation MP #1 with time is shown in Figure 5. Vertical displacement increases rapidly at the beginning of each load step due to the sudden increase of the applied load, then stabilizes approaching the quasi-static equilibrium.

Figure 6 shows the vertical displacement of the foundation at 6 seconds. Figure 7 shows the reaction forces of the foundation and the soil versus time. The maximum reaction force is 2000 kN/m.

Figure 5. Evolution of vertical displacement for a material point located in the foundation

Figure 6 Vertical displacement at t=6s

Figure 7 Reaction forces versus time