Benchmark Test: prescribedvelocityMP_contact_3D

written by Francesca Ceccato, 25/06/2021

General description

A cube of dry material is placed on the top of a 5m-dry soil column. The top material moves downward with a prescribed velocity (at the MP) compressing the bottom material. The moving mesh option is applied, the fix mesh is the cubical volume of prescribed velocity. The load multipliers for velocity are applied. The contact algorithm is applied.

Mesh

Size of discretized domain: 1 m x 1 m x 7 m

Type of element: tetrahedron

Size of elements: 1m

Num. of Tetrahedra elements=42

Num. of nodes=135

Quadratic meshNumber of MPs per element: 4 SMPs

Material properties

| $$MATERIAL_NAME bottom $$MATERIAL_TYPE dry_material $$POROSITY_SOLID 0.0 $$DENSITY_SOLID 1000.0 $$K0_VALUE_SOLID 0.0 $$MATERIAL_MODEL_SOLID linear_elasticity $$YOUNG_MODULUS 1000.0 $$MATERIAL_INDEX 2 $$MATERIAL_NAME top $$MATERIAL_TYPE dry_material $$POROSITY_SOLID 0.0 $$DENSITY_SOLID 1000.0 $$MATERIAL_MODEL_SOLID linear_elasticity $$YOUNG_MODULUS 1000.0 $$POISSON_RATIO 0.0 |  |
| --- | --- |

Boundary conditions

| The displacement at the boundaries will be ﬁxed in normal direction and free in the tangential directions. A prescribed velocity of -0.2m/s is prescribed in y-direction at the MP. |  |
| --- | --- |

Moving mesh is applied:

Gravity

Gravity is not applied

Contact

A smooth contact is applied

Numerical Solution:

### Anura3D_2018.2 ###

$$NUMBER_OF_LOADSTEPS

5

$$TIME_PER_LOADSTEP

0.1

$$TOTAL_TIME

0.0

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_ACCELERATION

9.81

$$GRAVITY_VECTOR

0.0 -1.0 0.0

$$GRAVITY_LOAD

off  0.0  1.0

$$SOLID_TRACTION

off  0.0  1.0

$$LIQUID_PRESSURE

off  0.0  1.0

$$PRESCRIBED_VELOCITY

step  1.0  1.0

$$STRAIN_SMOOTHING

0

$$CONTACT_FORMULATION

1

$$APPLY_OBJECTIVE_STRESS

0

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

5

$$OUTPUT_MATERIAL_POINTS

1

9

20

50

60

$$END

Solution

| step 1 (t=0.1s) | Step 5 (t=0.5s) |
| --- | --- |

Velocity of MP in the prescribed volume is -0.2m/s and the displacement respect the theoretical solution.

The moving mesh moves as expected.