Benchmark Test: Linear load_1 phase_linear elastic_2D:

written by Francesca Ceccato, 23/06/2021

General description

A dry soil column, with elastic behaviour, is loaded with a vertical load linearly variable along time. Bottom is fixed and the sample can only deform in vertical direction.

Mesh

Size of mesh: 0,1 m x 1,1 m x 0,1 m

Type of element: tetrahedron

Number of element: 66

Size of elements: 0.1

Number of nodes: 207

Number of MPs per element: 4 SMPs

Material properties

$$MATERIAL_NAME

Elastic

$$MATERIAL_TYPE

dry_material

$$POROSITY_SOLID

0.0

$$DENSITY_SOLID

1000

$$K0_VALUE_SOLID

0.0

$$MATERIAL_MODEL_SOLID

linear_elasticity

$$YOUNG_MODULUS

1000.0

$$POISSON_RATIO

0.0

Boundary conditions

The displacement at the boundaries will be ﬁxed in normal direction and free in the tangential directions

Initial conditions

A vertical load of -10 kPa is applied on the upper boundary, but it’s not suddenly applied: a linear increment of the load, till -10 kPa, is imposed in 3 steps  each one of 0,05 s.

Gravity

Gravity is not applied

Numerical Solution:

### Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

3

$$TIME_PER_LOADSTEP

0.05

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_LOAD

off  0.0  1.0

$$SOLID_TRACTION

linear  0.0  1.0

$$MASS_SCALING

0 1.0

$$HOMOGENEOUS_LOCAL_DAMPING

1 0.75

$$FIX_SOLID_SKELETON

0

$$STRAIN_SMOOTHING

0

$$APPLY_OBJECTIVE_STRESS

0

$$DEGREE_OF_FILLING

0.9

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

1

$$OUTPUT_MATERIAL_POINTS

1

$$END

The output is given for 1 material point; ID of the MP is 1.From the .PAR file it’s possible to plot time steps (or time) versus load multiplier.

Analytical Solution

No analytical solution