Benchmark Test: ColumnCollapse_AXI_MC_2Phase_K0_MPM-MP:

written by Luis A. Aviles, 21/07/2021

General description

A 2D axisymmetric saturated soil column is allowed to collapse by self-weight and run out on a rough surface. K0 procedure is used. no damping is considered. The soil run out in the horizontal direction over a surface. Material point integration is used (MPM-MP) and a single-point formulation.

Mesh

Size of mesh: 1,0 m x 1,0 m soil column – 3,0 m x 1,1 m total domain

Type of element: structured triangular_3-noded

Number of elements: 660

Size of elements: 0.1

Number of nodes: 372

Number of MPs per element: 3 MPs

Material properties

$$MATERIAL_NAME

soil

$$MATERIAL_TYPE

saturated_material_coupled

$$POROSITY_SOLID

0.40

$$DENSITY_SOLID

2650

$$DENSITY_LIQUID

1000

$$INTRINSIC_PERMEABILITY_LIQUID

1e-9

$$BULK_MODULUS_LIQUID

10000

$$DYNAMIC_VISCOSITY_LIQUID

1e-6

$$K0_VALUE_SOLID

0.5

$$MATERIAL_MODEL_SOLID

mohr_coulomb

$$YOUNG_MODULUS

1000

$$POISSON_RATIO

0.3

$$FRICTION_ANGLE

30.0

$$COHESION

0.0

$$DILATANCY_ANGLE

0.0

$$TENSILE_STRENGTH

0.0

Boundary conditions

The displacement for solid and liquid at the boundaries will be ﬁxed in normal direction and free in the tangential directions

Figure 1 Fixities applied in the model

Initial conditions

No initial conditions

Gravity

Gravity load is applied in the first-time step.

Numerical Solution:

### Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

50

$$TIME_PER_LOADSTEP

0.2

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MP

$$GRAVITY_LOAD

step  1.0  1.0

$$SOLID_TRACTION

step  0.0  1.0

$$LIQUID_PRESSURE

step  0.0  0.0

$$QUASISTATIC_CONVERGENCE

0

$$MASS_SCALING

0 1.0

$$HOMOGENEOUS_LOCAL_DAMPING

0 0.75

$$FIX_SOLID_SKELETON

0

$$STRAIN_SMOOTHING

0

$$K0_PROCEDURE

1

$$SURFACE_ELEVATION

1.0

$$INITIAL_VERTICAL_LOAD_K0

0.0

$$APPLY_OBJECTIVE_STRESS

0

$$DEGREE_OF_FILLING

0.9

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

5

$$OUTPUT_MATERIAL_POINTS

433

438

505

538

595

$$END

The output is given for 5 material point. From the .PAR file it is possible to plot time steps (or time) versus load multiplier.

Figure 2 points to control displacements

Figure 3 shows the horizontal displacements of the column.

Figure 3 horizontal displacements with time

Figure 4 horizontal displacements following the particles selected

Analytical Solution

No analytical solution