Benchmark Test: Oedometer_PorePressureTraction

written by Luis Aviles, 05/11/2021

General description

The benchmark consists of an elastic column under saturated fully coupled conditions. Uniform liquid pressure load of 5 kPa was applied to the top nodes (see figure 3). Gravitation loads are considered.

Mesh

Figure 1 geometry and discretization

$$DIMENSION

2D-plane_strain

$$ELEMENTTYPE

triangular_3-noded

number of MP per element = 3

Material properties

soil

$$MATERIAL_NAME Soil $$MATERIAL_TYPE saturated_material_coupled $$POROSITY_SOLID 0.6 $$DENSITY_SOLID 2650 $$DENSITY_LIQUID 1000 $$INTRINSIC_PERMEABILITY_LIQUID 1e-9 $$BULK_MODULUS_LIQUID 20000 $$DYNAMIC_VISCOSITY_LIQUID 1e-6 $$K0_VALUE_SOLID 0.5 $$MATERIAL_MODEL_SOLID linear_elasticity $$YOUNG_MODULUS 10000 $$POISSON_RATIO 0.3

Boundary conditions

Liquid and solid fixity in x and y direction are applied as shown in Figure 2

Figure 2 Boundary conditions

Loading conditions

A uniform pressure load of 5.0 kPa is applied at the top nodes of the column.

Figure 3 Liquid pressure loading conditions

Calculation parameters:

## Anura3D_2020 ###

$$NUMBER_OF_LOADSTEPS

10

$$TIME_PER_LOADSTEP

0.05

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_LOAD

linear  1.0  1.0

$$SOLID_TRACTION

off  0.0  1.0

$$LIQUID_PRESSURE

linear  0.0  1.0

$$PRESCRIBED_VELOCITY

off  0.0  1.0

$$K0_PROCEDURE

1

$$SURFACE_ELEVATION

1.0

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

5

$$OUTPUT_MATERIAL_POINTS

3

15

24

29

57

$$END

Solution

Figure 4 shows the pore pressure at the last time step as well as the mean effective stress.

Figure 4 MP mean effective stress and y-displacements