Benchmark Test: 2D applied pressure on MP

written by Francesca Ceccato, 29/05/2020 (updated: 09/09/2021)

General description

The benchmark considers an applied pressure at the MP on a circular surface. This is necessary to test loading boundary conditions for 2phase.

Mesh

Figure 1 geometry and discretization

$$DIMENSION

2D-plane_strain

$$ELEMENTTYPE

triangular_3-noded

number of MP per element = 3

Material properties

soil

$$MATERIAL_NAME Soil $$MATERIAL_TYPE saturated_material_coupled $$POROSITY_SOLID 0.4 $$DENSITY_SOLID 2650 $$DENSITY_LIQUID 1000 $$INTRINSIC_PERMEABILITY_LIQUID 1e-10 $$BULK_MODULUS_LIQUID 20000 $$DYNAMIC_VISCOSITY_LIQUID 1e-6 $$K0_VALUE_SOLID 0.0 $$MATERIAL_MODEL_SOLID linear_elasticity $$YOUNG_MODULUS 1000 $$POISSON_RATIO 0.0

Boundary conditions

Liquid fixity in x direction at symmetry axis

Loading conditions

A pressure load of -10kPa is applied at the MP on the hole.

Gravity is not applied.

Calculation parameters:

## Anura3D_2021 ###

$$NUMBER_OF_LOADSTEPS

10

$$TIME_PER_LOADSTEP

0.02

$$COURANT_NUMBER

0.98

$$COMPUTATION_METHOD

MPM-MIXED

$$GRAVITY_LOAD

off  0.0  1.0

$$SOLID_TRACTION

off  0.0  1.0

$$LIQUID_PRESSURE

step  1.0  1.0

$$PRESCRIBED_VELOCITY

off  0.0  1.0

$$FIX_SOLID_SKELETON

1

$$STRAIN_SMOOTHING

1

$$OUTPUT_NUMBER_OF_MATERIAL_POINTS

7

$$OUTPUT_MATERIAL_POINTS

88

114

424

427

616

747

874

$$FIX_SOLID_SKELETON

1

$$END

Solution

Figure 2 shows the pore pressure at the last time step. Figure 3 shows the external force liquid. The vector is correctly pointing normal to the surface.

| Figure 2 MP pressure at last time step | Figure 3 Arrows shows the direction of external force liquid |
| --- | --- |